LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: A & O RAILYARD

| Location: | End of North Kanawha and North Spring Streets Buckhannon, WV 26201 |
| --- | --- |
| Owner: | A & O Railroad 54 North Kanawha Street Buckhannon, WV 26201 |
| Emer. Contact: | Mr. Craig Youkum |
| Daytime Phone: | (304) 472-5690 |
| 24-hr. Phone: | (304) 613-1766 |
| First Due FD: | Buckhannon Fire Department |
| Daytime Phone: | 472-2868 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Diesel Fuel (click) |
| Lat./Long.: | 38.99692 / -80.23129 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) |

DETAILS – DIESEL FUEL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | 13,000 – 14,000 gallons on engines (no storage tanks on-site). |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, city streets, Wal-Mart, Sheetz, Burger King, Breckenridge, Upshur County Senior Center, Direct Care, First Central Bank, Rite Aid, Salvation Army, Buckhannon Manor, Southern States, downtown business district, multiple residences. |  |
| Susceptible Environment | Buckhannon River, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |