LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: AIRGAS MID AMERICAN (UPSHUR)

| Location: | Along US 33 (near Martin Oil fill station and Fleetwood Home Center) Buckhannon, WV 26201 |
| --- | --- |
| Owner: | Airgas Mid American P. O. Box 1117 Bowling Green, KY 42102 |
| Emer. Contact: | Mr. Jay Holsey |
| Daytime Phone: | (304) 269-1576 |
| 24-hr. Phone: | (304) 472-9214 |
| First Due FD: | Buckhannon Fire Department |
| Daytime Phone: | 472-2868 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Propane (click) |
| Lat./Long.: | 39.00176 / -80.2179 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) Aloha Analysis (click) |

DETAILS – PROPANE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Propane tank in yard, cylinders on dock. |  |
| Quantity | 10,000 – 99,999 lbs (max. up to 100,000 – 999,999 lbs) |  |
| Properties | Colorless gas with a faint petroleum odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1075 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi | Large Spills** Initial Isolation: 2,640 ft Daytime Isolation: 1.0 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, city streets, Cambridge Heights apartment complex, Fred Eberle Technical Center, Coastal Lumber, N. Buckhannon fire station, Fleetwood Homes, Martin Oil (Route 33 location), Heavner Cemetery, multiple residences. |  |
| Susceptible Environment | Buckhannon River, N. Buckhannon park, potential wetlands, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |