LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: ALTON PROJECT

| Location: | Near Alton and Selbyville Upshur County, West Virginia |
| --- | --- |
| Owner: | WV Department of Environmental Protection Route 2 Box 146-A Adrian, WV 26210 |
| Emer. Contact: | Mr. Wilbur Snider |
| Daytime Phone: | (304) 924-6231 |
| 24-hr. Phone: | (304) 924-6190 |
| First Due FD: | Adrian Volunteer Fire Department |
| Daytime Phone: | 472-4736 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Anhydrous Ammonia (click) Diesel Fuel (click) Sodium Hydroxide (click) |
| Lat./Long.: | 38.81794 / -80.20347 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) Aloha Analysis (click) |

DETAILS – ANHYDROUS AMMONIA

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | 12,000 and 9,200 gallon tanks throughout the complex. |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | A clear, colorless gas with a strong odor. |  |
| USDOT ID Number | 1005 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.4 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Facilities on the project grounds. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Vapors may cause irritation of the eyes and respiratory tract. Liquid will burn skin and eyes. Poisonous, may be fatal if inhaled. Contact with liquid may cause frostbite. May burn within certain vapor concentrations. |  |
| Exposure to Environment | Soluble in water with evolution of heat. The amount of heat generated may be large. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – DIESEL FUEL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | 2,000 and 500 gallon tanks at the office complex. |  |
| Quantity | 1,000 – 9,999 lbs (max. up to 10,000 – 99,999 lbs) |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Facilities on the project grounds. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – SODIUM HYDROXIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | 2,000, 3,000, and 4,000 gallon tanks throughout the complex. |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | Inhalation, ingestion, or skin contact may cause severe injury or death. Strong corrosive action on contacted tissues. |  |
| USDOT ID Number | 1824 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Facilities on the project grounds. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. Division ditch in place. |  |
| Exposure to People/ Property | Nearby residents and workers can fall victim to respiratory distress or death from windblown dust. Severe burns to eyes and skin may result with skin contact. When gases, liquid, and dust are given off, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control may cause pollution. Dust and runoff may cause suffocation or death of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |