LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: AMERICAN AGIP COMPANY, INC.

| Location: | Lewis County Industrial Park Jane Lew, WV 26378 |
| --- | --- |
| Owner: | American Agip Company, Inc. 485 Madison Avenue New York, NY 10022 |
| Emer. Contact: | Mr. Michael Hicks |
| Daytime Phone: | (304) 884-8210 |
| 24-hr. Phone: | (304) 266-2097 |
| First Due FD: | Jane Lew Volunteer Fire Department |
| Daytime Phone: | 884-7231 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Diesel Fuel (click) Ethylene Glycol (click) |
| Lat./Long.: | 39.1013 / -80.39749 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) Aloha Analysis (click) |

DETAILS – DIESEL FUEL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Block building behind warehouse. |  |
| Quantity | 1,000 – 9,999 lbs (max. up to 10,000 – 99,999 lbs) |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | City streets, Dominion, PTC Alliance, Linn Operating, Viking Pools, CPC, banking facility. |  |
| Susceptible Environment | Industrial park grounds. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – ETHYLENE GLYCOL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | In warehouse. |  |
| Quantity | 1,000 – 9,999 lbs (max. up to 10,000 – 99,999 lbs) |  |
| Properties | Clear, colorless syrupy liquid. Primary hazard is a threat to the environment. Take immediate steps to limit spread. Can easily penetrate soil and contaminate groundwater and nearby streams. |  |
| USDOT ID Number | 1171 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 2,640 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | City streets, highways (including I-79), Worldwide Equipment, Dominion, PTC Alliance, Linn Operating, Viking Pools, CPC, banking facility, post office, Go Mart, Jane Lew Truck Stop, Plantation Inn, miscellaneous residences. |  |
| Susceptible Environment | Hackers Creek, wetland areas along I-79. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Irritating to skin and eyes. Can cause respiratory irritation, dizziness, nausea, and unconsciousness. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |