LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: BUCKHANNON WATER TREATMENT PLANT

| Location: | Wood Street Buckhannon, WV 26201 |
| --- | --- |
| Owner: | City of Buckhannon 70 East Main Street Buckhannon, WV 26201 (304) 472-1430 |
| Emer. Contact: | Mr. Burl Smith, City Engineer |
| Daytime Phone: | (304) 472-1430 |
| 24-hr. Phone: | (304) 472-8776 |
| First Due FD: | Buckhannon Fire Department |
| Daytime Phone: | 472-2868 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Chlorine (click) Potassium Permanganate (click) |
| Lat./Long.: | 38.98042 / -80.21976 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) Aloha Analysis (click) |

DETAILS – CHLORINE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Chlorine Storage Room |  |
| Quantity | 1,000 – 9,999 lbs |  |
| Properties | A greenish yellow gas with a pungent suffocating odor. Toxic by inhalation. Slightly soluble in water. Liquefies at -35°C and room pressure. Readily liquefied by pressure applied at room temperature. Contact with unconfined liquid can cause frostbite by evaporative cooling. Does not burn but, like oxygen, supports combustion. |  |
| USDOT ID Number | 1017 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 800 ft Daytime Isolation: 1.5 mi Nighttime Isolation: 4.6 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | City streets, A & O railroad, A.F. Wendling storage facility, West Buckhannon Baptist Church, multiple residences. |  |
| Susceptible Environment | Buckhannon River and county public water supply. Possibly nearby farm animals. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Nearby residents and workers can fall victim to respiratory distress or death from wind-blown vapors. When gases and liquid are given off, equipment, vehicles, etc. susceptible to corrosion may be affected. They would be repairable, however. |  |
| Exposure to Environment | Possible destruction of surrounding fauna and flora. Potential contamination of Buckhannon River. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | Within 100-year floodplain. |  |

DETAILS – POTASSIUM PERMANGANATE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Chemical Feed Room |  |
| Quantity | 100 – 999 lbs |  |
| Properties | A purplish colored crystalline solid. Noncombustible but accelerates the burning of combustible material. If the combustible material is finely divided the mixture may be explosive. Contact with liquid combustible materials may result in spontaneous ignition. Contact with sulfuric acid may cause fire or explosion. Used to make other chemicals and as a disinfectant. |  |
| USDOT ID Number | 1490 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | City streets, A & O railroad, A.F. Wendling storage facility, multiple residences. |  |
| Susceptible Environment | Buckhannon River and county public water supply. Possibly nearby farm animals. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Nearby residents and workers can fall victim to respiratory distress, burns, or death from vapors. If gases are given off by fire, equipment, vehicles, etc., a susceptibility to corrosion may be present (repairable). |  |
| Exposure to Environment | Possible destruction of surrounding fauna and flora. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | Within 100-year floodplain. |  |