LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: HALLIBURTON

| Location: | 1628 Jackson’s Mill Road (near country club) Jane Lew, WV 26378 |
| --- | --- |
| Owner: | Halliburton 10200 Bellaire Boulevard Houston, TX 77072 |
| Emer. Contact: | Mr. Michael Wilson |
| Daytime Phone: | (724) 479-4266 |
| 24-hr. Phone: | (888) 223-4255 |
| First Due FD: | Jackson’s Mill Volunteer Fire Department |
| Daytime Phone: | 269-5158 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Calcium Chloride (click) Cement (click) Hydrochloric Acid (click) Liquefied Nitrogen (click) Silicalite (click) *various other trade-specific materials |
| Lat./Long.: | 39.08536 / -80.46579 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) Aloha Analysis (click) |

DETAILS – CALCIUM CHLORIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | IIII primary stock, 1005 SO consignment. |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | White to off-white solid. Sinks and mixes with water. |  |
| USDOT ID Number | N/A |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Halliburton equipment and facilities, slight potential to affect neighboring Deerfield Country Club. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Inhalation causes irritation of nose and throat. Ingestion causes irritation of mouth and stomach. Contact with eyes (particularly by dust) causes irritation and possible transient corneal injury. Contact of solid with dry skin causes mild irritation; strong solutions can cause marked irritation, even a superficial burn. |  |
| Exposure to Environment | Adding calcium chloride to hot water may cause boiling. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – PORTLAND CEMENT

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | IIII primary stock, 1005 SO consignment |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | Gray or white powder with no odor. |  |
| USDOT ID Number | N/A |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Halliburton equipment and facilities, slight potential to affect neighboring Deerfield Country Club. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Dust can irritate eyes and upper respiratory tract. Wet cement may cause alkali burns to the skin. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – HYDROCHLORIC ACID

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | IIII primary stock, 1005 SO consignment. |  |
| Quantity | 10,000 – 99,999 lbs (max. up to 100,000 – 999,999 lbs) |  |
| Properties | Colorless, watery liquid with a sharp, irritating odor. |  |
| USDOT ID Number | 1789 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Roadways, Halliburton equipment and facilities, potential exists to impact portions of the Deerfield Country Club, neighboring commercial structures. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Inhalation of fumes causes coughing and choking sensation and irritation of the nose and lungs. Contact with the liquid causes burns. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution and may be corrosive or toxic. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – LIQUEFIED NITROGEN

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | IIII primary stock |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | Colorless, odorless liquid. Very cold. |  |
| USDOT ID Number | 1977 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Roadways, Halliburton equipment and facilities, potential exists to impact portions of the Deerfield Country Club, neighboring commercial structures. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Contact of liquid with skin or eyes causes frostbite burns. Inhalation can cause asphyxiation, if atmosphere does not contain oxygen; dizziness, unconsciousness, or even death can result. Prolonged contact to equipment, etc. may cause damage. Containers may explode when burning. |  |
| Exposure to Environment | No rapid reaction with air or water. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – SILICALITE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | IIII primary stock. |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | Transparent to gray, odorless powder. |  |
| USDOT ID Number | N/A |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Halliburton equipment and facilities, slight potential to affect neighboring Deerfield Country Club. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Irritating to the skin and eyes on contact. Inhalation will cause irritation in the respiratory tract. |  |
| Exposure to Environment | No rapid reaction with air or water. |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | None known. |  |