LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

INTERSTATE 79 (EXIT 99)

| Location: | Just east of Weston near Wal-Mart and Market Place Weston, WV 26452 |  |
| --- | --- | --- |
| Owner: | N/A |  |
| Emer. Contact: | N/A |  |
| Daytime Phone: | N/A |  |
| 24-hr. Phone: | N/A |  |
| First Due FD: | Weston Fire Department |  |
| Daytime Phone: | 269-2349 |  |
| 24-hr. Phone: | 911 |  |
| History of Events: | None to report |  |
| Material(s): | Ammonium Nitrate (click) Anhydrous Ammonia (click) Butyl Acrylate (click) Carbon Dioxide (click) Chlorine (click) Cumene (click) Cyclohexylamine (click) Diesel Fuel (click) Dimethylaminoethyl Acrylate (click) Gasoline (click) Hexaldehyde (click) Hydrochloric Acid (click) Hydrogen Peroxide (click) Liquid Nitrogen (click) Magnesium Diphenyl (click) | Maleic Anhydride (click) Methylethyl Ketone Methyl Methacrylate Monomer (click) Neon (click) Organic Peroxide Oxygen (click) Paint (click) Pesticide Phenylenediamines (click) Phosphorus Trichloride (click) Polymeric Beads (click) Propane (click) Sulfur Dioxide (click) Sulfuric Acid (click) Thallium (click) |
| Lat./Long.: | 39.02176 / -80.42757 |  |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) |  |

DETAILS – AMMONIUM NITRATE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless, crystalline solid. |  |
| USDOT ID Number | 1942 |  |
| Isolation Distances | Small Spills* Initial Isolation: 75 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 100 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | No health information available. Accelerates the burning of combustible material. Produces toxic oxides of nitrogen during combustion. |  |
| Exposure to Environment | Water soluble. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – ANHYDROUS AMMONIA

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A clear, colorless gas with a strong odor. |  |
| USDOT ID Number | 1005 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.4 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Vapors may cause irritation of the eyes and respiratory tract. Liquid will burn skin and eyes. Poisonous, may be fatal if inhaled. Contact with liquid may cause frostbite. May burn within certain vapor concentrations. |  |
| Exposure to Environment | Soluble in water with evolution of heat. The amount of heat generated may be large. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – BUTYL ACRYLATE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A clear, colorless liquid with a sharp characteristic odor. |  |
| USDOT ID Number | 2348 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Vapor is irritating when inhaled at high concentrations. Contact with liquid causes irritation of skin and burning of eyes. Highly flammable. Vapor explosion hazard indoors or in sewers. |  |
| Exposure to Environment | Vapors may form explosive mixtures with air. Vapors will spread along the ground and collect in low or confined areas such as sewers, basements, and/or tanks. Liquid is lighter than water. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – CARBON DIOXIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless, odorless gas at atmospheric temperatures and pressures. |  |
| USDOT ID Number | 1013 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Inhalation causes increased respiration rate, headache, and subtle psychological changes. High concentrations can cause unconsciousness and death. Solid can cause cold-contact burns. Liquid or cold gas can cause freezing injury to skin or eyes similar to a burn. Containers may explode when heated. |  |
| Exposure to Environment | Water soluble, may form acidic solution in water. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – CHLORINE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A greenish yellow gas with a pungent suffocating odor. Toxic by inhalation. Slightly soluble in water. Liquefies at -35°C and room pressure. Readily liquefied by pressure applied at room temperature. Contact with unconfined liquid can cause frostbite by evaporative cooling. Does not burn but, like oxygen, supports combustion. |  |
| USDOT ID Number | 1017 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 800 ft Daytime Isolation: 1.5 mi Nighttime Isolation: 4.6 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Nearby residents and workers can fall victim to respiratory distress or death from wind-blown vapors. When gases and liquid are given off, equipment, vehicles, etc. susceptible to corrosion may be affected. They would be repairable, however. |  |
| Exposure to Environment | Possible destruction of surrounding fauna and flora. Potential contamination of Buckhannon River. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – CUMENE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A clear, colorless liquid with an aromatic odor. |  |
| USDOT ID Number | 1918 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Narcotic action with long-lasting effects; depressant to central nervous system. Flammable. Containers may explode when heated. |  |
| Exposure to Environment | Vapors may form explosive mixtures with air. Vapors will spread along the ground and collect in low or confined areas such as sewers, basements, and/or tanks. Liquid is lighter than water. Runoff to sewer may create fire or explosion hazard. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – CYCLOHEXYLAMINE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A clear, colorless to yellow liquid with an odor of ammonia. |  |
| USDOT ID Number | 2357 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Emits toxic fumes when heated. Vapor may travel a considerable distance from source of ignition and flash back. Very toxic – lethal oral dose for 150 lb. person is approximately 1 teaspoon. Cyclohexylamine is considered a nerve poison. |  |
| Exposure to Environment | Highly flammable. Sensitive to air and light. Soluble in water. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – DIESEL FUEL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – DIMETHYLAMINOETHYL ACRYLATE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless to light yellow liquid with an acrid odor. |  |
| USDOT ID Number | 3302 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.3 mi Nighttime Isolation: 1.4 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Irritates eyes and produces tears. Highly toxic; may be fatal if inhaled, swallowed, or absorbed through skin. Contact with molten substance may cause severe burns to skin and eyes. |  |
| Exposure to Environment | Soluble in water; hydrolyzes in water to produce acrylic acid and dimethylaminoethanol. Fire may produce irritating, corrosive, or toxic gases. Runoff from fire control or dilution water may be toxic or corrosive and cause pollution. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – GASOLINE (UNLEADED)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Clear to amber-colored volatile liquid with a petroleum-like odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewers may create fire or explosion hazards. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1203 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – HEXALDEHYDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A clear, colorless liquid with a pungent odor. |  |
| USDOT ID Number | 1207 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Ingestion causes irritation of the mouth and stomach. Contact with vapor or liquid irritates eyes. Liquid irritates skin. |  |
| Exposure to Environment | Highly flammable. Insoluble in water. |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – HYDROCHLORIC ACID

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Colorless, watery liquid with a sharp, irritating odor. |  |
| USDOT ID Number | 1789 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Inhalation of fumes causes coughing and choking sensation and irritation of the nose and lungs. Contact with the liquid causes burns. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution and may be corrosive or toxic. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – HYDROGEN PEROXIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A crystalline solid at low temperatures; has a slightly pungent, irritating odor. |  |
| USDOT ID Number | 2015 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Strong irritate to eyes, skin, and mucous membranes. Spontaneous ignition may occur when in contact with combustible materials. Severe explosion hazard when it is exposed to heat, mechanical impact, detonation of a blasting cap, or caused to decompose catalytically. |  |
| Exposure to Environment | Severe explosion hazard when it is exposed to heat, mechanical impact, detonation of a blasting cap, or caused to decompose catalytically. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – LIQUID NITROGEN

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Colorless, odorless liquid. Very cold. |  |
| USDOT ID Number | 1977 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Contact of liquid with skin or eyes causes frostbite burns. Inhalation can cause asphyxiation, if atmosphere does not contain oxygen; dizziness, unconsciousness, or even death can result. Prolonged contact to equipment, etc. may cause damage. Containers may explode when burning. |  |
| Exposure to Environment | No rapid reaction with air or water. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – MAGNESIUM DIPHENYL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A whitish to gray crystalline solid. |  |
| USDOT ID Number | 2005 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.3 mi Nighttime Isolation: 1.4 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Fire produces irritating, corrosive, or toxic gases. Inhalation of decomposition products may cause severe injury or death. Contact with substance may cause severe burns to skin and eyes. |  |
| Exposure to Environment | Flammable/combustible material. May ignite on contact with moist air or moisture. Runoff from fire control may cause pollution. Runoff may create fire or explosion hazard. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – MALEIC ANHYDRIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Colorless crystalline needles, flakes, pellets, rods, briquettes, lumps or a fused mass. |  |
| USDOT ID Number | 2215 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.3 mi Nighttime Isolation: 1.4 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Inhalation causes coughing, sneezing, throat irritation. Skin contact causes irritation and redness. Vapors cause severe eye irritation; photophobia and double vision may occur. |  |
| Exposure to Environment | Soluble in water. Reacts slowly with water to form maleic acid and heat. Reacts exothermically with water or steam. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – METHYLETHYL KETONE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A clear, colorless liquid of high viscosity. |  |
| USDOT ID Number | 1193 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Severely irritating to eyes; if not removed promptly, may cause permanent damage. Skin contact causes irritation. Vapors are irritating to the respiratory tract, may cause pulmonary edema, and may affect the central nervous system. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – METHYL METHACRYLATE MONOMER

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A clear, colorless liquid. |  |
| USDOT ID Number | 1247 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | May cause toxic effects if inhaled or absorbed through skin. Inhalation or contact with material may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. |  |
| Exposure to Environment | Highly flammable; will be easily ignited by heat, sparks or flames. Vapors may form explosive mixtures with air. Runoff from fire control or dilution water may cause pollution. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – NEON

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless odorless noncombustible gas. |  |
| USDOT ID Number | 1065 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Vapors may cause dizziness or asphyxiation without warning. |  |
| Exposure to Environment | Vapors from liquefied gas are initially heavier than air and spread along ground. |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – ORGANIC PEROXIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | N/A |  |
| USDOT ID Number | 3110 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 800 ft Daytime Isolation: 1.5 mi Nighttime Isolation: 2.4 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | N/A |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – OXYGEN

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless, odorless, tasteless gas. |  |
| USDOT ID Number | 1072 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 1,760 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Super 8, cemetery, Weston Assembly of God, Jordan Truck Parts, multiple residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Liquid may cause frostbite of eyes and skin. Increases intensity of any fire. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – PAINT

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A liquid with a petroleum-like odor. |  |
| USDOT ID Number | 1263 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Inhalation or contact with material may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. |  |
| Exposure to Environment | Fire may produce irritating, corrosive and/or toxic gases. Runoff from fire control or dilution water may cause pollution. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – PESTICIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | N/A |  |
| USDOT ID Number | 3007 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 300 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | N/A |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – PHENYLENEDIAMINES

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | White to light purple crystals. |  |
| USDOT ID Number | 1673 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 300 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Poisonous by ingestion. Mildly toxic by skin contact. Contact with molten substance may burn eyes or skin. |  |
| Exposure to Environment | Slowly oxidizes on standing air. Fire may produce irritating, corrosive and/or toxic gases. Runoff from fire control or dilution water may be corrosive and/or toxic and cause pollution. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – PHOSPHORUS TRICHLORIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless or slightly yellow fuming liquid with a pungent and irritating odor resembling that of hydrochloric acid. |  |
| USDOT ID Number | 1809 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 500 ft Daytime Isolation: 1.0 mi Nighttime Isolation: 2.2 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Highly toxic; it may cause death or permanent injury. Contact is highly irritating to the skin, eyes, and mucous membranes, and the material is an irritant through oral and inhalation exposure. |  |
| Exposure to Environment | Fumes in air. Reacts violently with water, producing toxic/corrosive fumes. Hydrolysis produces dense hydrogen chloride vapors. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – POLYMERIC BEADS

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Insoluble in water and less dense than water. |  |
| USDOT ID Number | 2211 |  |
| Isolation Distances | Small Spills* Initial Isolation: 75 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Contact may cause burns to skin and eyes. Contact with molten substance may cause severe burns to skin and eyes. |  |
| Exposure to Environment | Flammable. Insoluble in water. Fire may produce irritating and/or toxic gases. Runoff from fire control may cause pollution. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – PROPANE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Colorless gas with a faint petroleum odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1075 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi | Large Spills** Initial Isolation: 2,640 ft Daytime Isolation: 1.0 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, all shops within Market Place, McDonalds, Progressive Bank, Steer Steakhouse, Super 8, cemetery, Weston Assembly of God, Jordan Truck Parts, Twin Lakes Camper Sales, multiple residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – SULFUR DIOXIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless gas with a choking or suffocating odor. |  |
| USDOT ID Number | 1079 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 700 ft Daytime Isolation: 1.3 mi Nighttime Isolation: 3.9 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | May cause death or permanent injury after very short exposure to small quantities. 1,000 ppm causes death in from 10 minutes to several hours by respiratory depression. It is an eye and respiratory tract irritant. Persons with asthma, subnormal pulmonary functions or cardiovascular disease are at a greater risk. |  |
| Exposure to Environment | Dissolves in water to form sulfurous acid, a corrosive liquid. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – SULFURIC ACID

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Colorless, oily liquid. |  |
| USDOT ID Number | 1830 |  |
| Isolation Distances | Small Spills* Initial Isolation: 200 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WVDOH garage, Weston Assembly of God, residences. |  |
| Susceptible Environment | Stonecoal Creek, terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Corrosive to all body tissues. Inhalation of vapor may cause serious lung damage. Contact with eyes may result in total loss of vision. Skin contact may produce severe necrosis. Fatal amount for adult: between 1 teaspoonful and one-half ounce of the concentrated chemical. Even a few drops may be fatal if the acid gains access to the trachea. Chronic exposure may cause tracheobronchitis, stomatitis, conjunctivitis, and gastritis. Gastric perforation and peritonitis may occur and may be followed by circulatory collapse. Circulatory shock is often the immediate cause of death. Those with chronic respiratory, gastrointestinal, or nervous diseases and any eye and skin diseases are at greater risk. It is corrosive to metals and tissue. It will char wood and most other organic matter on contact, but is unlikely to cause a fire. Density 15 lb / gal. Long term exposure to low concentrations or short term exposure to high concentrations can result in adverse health effects from inhalation. Immediate onset and persistent for hours or days. |  |
| Exposure to Environment | Soluble in water with release of heat. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – THALLIUM

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Variably colored compounds containing thallium. |  |
| USDOT ID Number | 1707 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 300 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Highly toxic; may be fatal if inhaled, swallowed, or absorbed through skin (effects may be delayed). Fire may produce irritating, corrosive, and/or toxic gases. |  |
| Exposure to Environment | May or may not be water soluble. Runoff from fire control or dilution water may be corrosive and/or toxic and cause pollution. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |