LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: JF ALLEN LORENTZ PLANT

| Location: | Route 33 West (just off on Sauls Run Road) Buckhannon, WV 26201 |
| --- | --- |
| Owner: | J.F. Allen Company Route 33 West on Route 151 Buckhannon, WV 26201 (304) 472-9586 |
| Emer. Contact: | Mr. Michael Griffith |
| Daytime Phone: | (304) 472-8890 |
| 24-hr. Phone: | (304) 614-3175 |
| First Due FD: | Buckhannon Fire Department |
| Daytime Phone: | 472-2868 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Asphalt Emulsion Diesel Fuel (click) Fly Ash Gasoline – Unleaded (click) Limestone Dust Liquefied Petroleum Asphalt Portland Cement (click) |
| Lat./Long.: | 39.00789 / -80.30983 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) |

DETAILS – ASPHALT EMULSION

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Tank trailers at northwest end of property |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Brown to black liquid or semi liquid. |  |
| USDOT ID Number | 1999 (Asphalt) |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 300 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | JF Allen facilities and equipment. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Heated product may irritate eyes, skin, and lungs. Can cause thermal burns. Unheated asphalt may be irritating. |  |
| Exposure to Environment | Fires will produce irritating, corrosive, and/or toxic gases. Equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | Portions of the property are within the 100-year floodplain; however, no facilities are within the floodplain. |  |

DETAILS – DIESEL FUEL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Tank at north side of asphalt plant and tank at pump island |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | JF Allen facilities and equipment, residences. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | Portions of the property are within the 100-year floodplain; however, no facilities are within the floodplain. |  |

DETAILS – FLY ASH

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Silo at concrete plant |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Odorless, light gray/tan to dark gray powder. |  |
| USDOT ID Number | N/A |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | JF Allen facilities and equipment. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Inhalation may cause nose, throat, and lung irritation and choking. May cause skin and eye irritation. Ingestion may cause intestinal distress. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | Portions of the property are within the 100-year floodplain; however, no facilities are within the floodplain. |  |

DETAILS – GASOLINE (UNLEADED)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Tank at pump island |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Clear to amber-colored volatile liquid with a petroleum-like odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewers may create fire or explosion hazards. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1203 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | JF Allen facilities and equipment, residences. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | Portions of the property are within the 100-year floodplain; however, no facilities are within the floodplain. |  |

DETAILS – LIMESTONE DUST

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Silo at asphalt plant |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | White powder |  |
| USDOT ID Number | N/A |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | JF Allen facilities and equipment. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | An irritant to the eyes and skin. Effects of inhalation or contact may be delayed. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | Portions of the property are within the 100-year floodplain; however, no facilities are within the floodplain. |  |

DETAILS – LIQUEFIED PETROLEUM ASPHALT

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Tanks at asphalt plant |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | Brown to black liquid or semi-liquid. |  |
| USDOT ID Number | N/A |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | JF Allen facilities and equipment. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Heated product may irritate eyes, skin, and lungs. Can cause thermal burns. Unheated asphalt may be irritating. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation in fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | Portions of the property are within the 100-year floodplain; however, no facilities are within the floodplain. |  |

DETAILS – PORTLAND CEMENT

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Silo at concrete plant |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | Gray or white powder with no odor. |  |
| USDOT ID Number | N/A |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | JF Allen facilities and equipment. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Dust can irritate eyes and upper respiratory tract. Wet cement may cause alkali burns to the skin. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | Portions of the property are within the 100-year floodplain; however, no facilities are within the floodplain. |  |