LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: MARTIN OIL

| Location: | Old Route 33 in Deanville (near Superior Well Service) and along US 33 near Fleetwood Homes Buckhannon, WV 26201 |
| --- | --- |
| Owner: | Martin Oil Company, Inc. P. O. Box 78 Buckhannon, WV 26201 |
| Emer. Contact: | Mr. C.J. Martin, Owner |
| Daytime Phone: | (304) 472-1184 |
| 24-hr. Phone: | (304) 472-1363 |
| First Due FD: | Buckhannon Fire Department |
| Daytime Phone: | 472-2868 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Diesel Fuel (click) Gasoline, Unleaded (click) Methanol (click) Petroleum Products (click) |
| Lat./Long.: | 38.9897 / -80.2044 (Deanville) 39.00281 / -80.2206 (Route 33) |
| Additional Links: | Site Map (Deanville – Route 33) Initial Isolation Distances (Deanville – Route 33) Aloha Analysis (Deanville) |

DETAILS – DIESEL FUEL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Diked bulk area at Deanville location; bulk tanks at US 33 location. |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, city streets, Fleetwood Homes, Airgas, Superior Well Service, Rite Way, Micrologic, Buckhannon Lanes, churches, multiple residences. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known (either location). |  |

DETAILS – GASOLINE (UNLEADED)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Diked bulk area at Deanville location; bulk tanks at US 33 location. |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | Clear to amber-colored volatile liquid with a petroleum-like odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewers may create fire or explosion hazards. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1203 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, city streets, Fleetwood Homes, Airgas, Superior Well Service, Rite Way, Micrologic, Buckhannon Lanes, churches, multiple residences. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known (either location). |  |

DETAILS – METHANOL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Warehouse building across from main office building |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Colorless, volatile liquid with a faintly sweet pungent smell like ethyl alcohol. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1230 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, city streets, Fleetwood Homes, Airgas, Superior Well Service, Rite Way, Micrologic, Buckhannon Lanes, churches, multiple residences. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known (either location). |  |

DETAILS – PETROLEUM PRODUCTS, N.O.S. (Petroleum Ether)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Warehouse at Deanville location. |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Colorless liquid with gasoline or kerosene-like odor. Comprised of a mixture of paraffins and may contain aromatic hydrocarbons. |  |
| USDOT ID Number | 1255 or 1271 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, city streets, Fleetwood Homes, Airgas, Superior Well Service, Rite Way, Micrologic, Buckhannon Lanes, churches, multiple residences. |  |
| Susceptible Environment | Wooded, rural area – terrestrial life, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Highly flammable and vapors may form with mixtures in air. Vapor irritates respiratory tract, causes coughing and mild depression. Aspiration causes severe lung irritation with coughing, gagging, and rapidly-developing pulmonary edema. Ingestion irritates mouth and stomach. May be ignited by strong oxidizers. |  |
| Exposure to Environment | Runoff to sewer may cause fire or explosion hazard. Many liquids are lighter than water. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known (either location). |  |