LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: PTC ALLIANCE

| Location: | 533 Industrial Park Road (Lewis County Industrial Park) Jane Lew, WV 26378 |
| --- | --- |
| Owner: | PTC Alliance 5061 Wallace Road Extension, Suite 200 Wexford, PA 15090 |
| Emer. Contact: | Mr. Randy Bell |
| Daytime Phone: | (304) 884-7801 |
| 24-hr. Phone: | N/A |
| First Due FD: | Jane Lew Volunteer Fire Department |
| Daytime Phone: | 884-7231 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Petroleum Products (click) Sulfuric Acid (click) |
| Lat./Long.: | 39.10063 / -80.39507 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) |

DETAILS – PETROLEUM PRODUCTS, N.O.S. (Petroleum Ether)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Oil dip tank area, fuel oil tank area, miscellaneous storage area. |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Colorless liquid with gasoline or kerosene-like odor. Comprised of a mixture of paraffins and may contain aromatic hydrocarbons. |  |
| USDOT ID Number | 1255 or 1271 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, Viking Pools/CPC, American Agip, Linn Operating, Worldwide Equipment, Dominion, other industrial park facilities. |  |
| Susceptible Environment | Low-lying areas along I-79 wherein flood waters collect during significant rain events. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Highly flammable and vapors may form with mixtures in air. Vapor irritates respiratory tract, causes coughing and mild depression. Aspiration causes severe lung irritation with coughing, gagging, and rapidly-developing pulmonary edema. Ingestion irritates mouth and stomach. May be ignited by strong oxidizers. |  |
| Exposure to Environment | Runoff to sewer may cause fire or explosion hazard. Many liquids are lighter than water. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – SULFURIC ACID

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Spent acid tank in northeast corner of building, in the pickling tank in the pickle area, acid rinse tank. |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Colorless, oily liquid. |  |
| USDOT ID Number | 1830 |  |
| Isolation Distances | Small Spills* Initial Isolation: 200 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, Viking Pools/CPC, American Agip, Linn Operating, Worldwide Equipment, Dominion, other industrial park facilities. |  |
| Susceptible Environment | Low-lying areas along I-79 wherein flood waters collect during significant rain events. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Corrosive to all body tissues. Inhalation of vapor may cause serious lung damage. Contact with eyes may result in total loss of vision. Skin contact may produce severe necrosis. Fatal amount for adult: between 1 teaspoonful and one-half ounce of the concentrated chemical. Even a few drops may be fatal if the acid gains access to the trachea. Chronic exposure may cause tracheobronchitis, stomatitis, conjunctivitis, and gastritis. Gastric perforation and peritonitis may occur and may be followed by circulatory collapse. Circulatory shock is often the immediate cause of death. Those with chronic respiratory, gastrointestinal, or nervous diseases and any eye and skin diseases are at greater risk. It is corrosive to metals and tissue. It will char wood and most other organic matter on contact, but is unlikely to cause a fire. Density 15 lb / gal. Long term exposure to low concentrations or short term exposure to high concentrations can result in adverse health effects from inhalation. Immediate onset and persistent for hours or days. |  |
| Exposure to Environment | Soluble in water with release of heat. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |