LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: STONEWALL JACKSON MEMORIAL HOSPITAL

| Location: | 230 Hospital Plaza (just west of Weston along US 33) Weston, WV 26452 |
| --- | --- |
| Owner: | Stonewall Jackson Memorial Hospital 230 Hospital Plaza Weston, WV 26452 |
| Emer. Contact: | Mr. Rick Simpson |
| Daytime Phone: | (304) 269-8131 |
| 24-hr. Phone: | (304) 452-9445 |
| First Due FD: | Weston Fire Department |
| Daytime Phone: | 269-2349 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | #2 Fuel Oil (click) Liquid Oxygen (click) |
| Lat./Long.: | 39.0483 / -80.49247 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) |

DETAILS – #2 FUEL OIL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Two (2) 500 gal. above ground tanks at receiving area behind main building. |  |
| Quantity | 100 – 999 lbs |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1202 or 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, city streets, hospital facilities, multiple residences. |  |
| Susceptible Environment | Terrestrial life – urban and rural. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – LIQUID OXYGEN

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Receiving area behind main building. |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Colorless, odorless, tasteless gas. Non-combustible, but will support the burning of combustible materials. |  |
| USDOT ID Number | 1073 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 1,760 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, city streets, Citgo station, health clinic, hospital facilities, multiple residences. |  |
| Susceptible Environment | Terrestrial life – urban and rural. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | N/A |  |
| Exposure to Environment | Non reactive in air and water. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |