LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: STONEWALL RESORT

| Location: | 940 Resort Drive (south of Weston off of US Route 19) Roanoke, WV 26447 |
| --- | --- |
| Owner: | WV Division of Natural Resources 149 State Park Trail Roanoke, WV 26447 |
| Emer. Contact: | Mr. Floyd Hunter |
| Daytime Phone: | (304) 269-7400 |
| 24-hr. Phone: | (304) 269-7400 |
| First Due FD: | Walkersville Volunteer Fire Department |
| Daytime Phone: | 452-8451 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Gasoline (click) |
| Lat./Long.: | 38.44262 / -80.4947 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) |

DETAILS – GASOLINE (UNLEADED)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | In a below-ground tank at the marina and in above-ground tanks at the golf pro shop and golf maintenance shop. |  |
| Quantity | 1,000-9,999 lbs (max. up to 10,000 – 99,999 lbs) |  |
| Properties | Clear to amber-colored volatile liquid with a petroleum-like odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewers may create fire or explosion hazards. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1203 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Resort/park roadways, lodge facility, marina. |  |
| Susceptible Environment | Stonewall Jackson Lake, resort/park grounds. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |