LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

US 33 / SR 20 INTERSECTION

| Location: | Just north of Buckhannon (between Wal-Mart and Lowe’s) Buckhannon, WV 26201 |
| --- | --- |
| Owner: | N/A |
| Emer. Contact: | N/A |
| Daytime Phone: | N/A |
| 24-hr. Phone: | N/A |
| First Due FD: | Buckhannon Fire Department |
| Daytime Phone: | 472-2868 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Crude Oil (click) Dichlorodimethyl Ether (click) Diesel Fuel (click) Difluorophosphoric Acid, Anhydrous (click) Di-n-Butylamine (click) Gasoline (click) Lead Dioxide (click) Liquid Nitrogen (click) Oxygen (click) Propane (click) Resin Solution (click) Stannic Phosphide (click) |
| Lat./Long.: | 39.00137 / -80.23251 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) |

DETAILS – CRUDE OIL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A dark viscous liquid. |  |
| USDOT ID Number | 1267 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, electric substation, Sheetz, Fast Stop Exxon, Record Delta office, Brake’s Dairy King, Dollar Tree, Breckenridge equipment lot, Pizza Hut, Lowe’s. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | N/A |  |
| Exposure to Environment | Flammable. Insoluble in water. |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – DICHLORODIMETHYL ETHER

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless volatile liquid with a chloroform-like odor. |  |
| USDOT ID Number | 2249 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 300 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Acute toxicity is high by ingestion, inhalation, and skin irritation. Small quantities may cause death or permanent injury after very short exposure. Chloromethyl ether is an alkylating agent which is a recognized human carcinogen. There is a strong association between industrial exposure and excess lung cancer. |  |
| Exposure to Environment | Highly flammable. Insoluble in water. Reacts with water to form hydrochloric acid & formaldehyde. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – DIESEL FUEL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, electric substation, Sheetz, Fast Stop Exxon, Record Delta office, Brake’s Dairy King, Dollar Tree, Breckenridge equipment lot, Pizza Hut, Lowe’s. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – DIFLUOROPHOSPHORIC ACID, ANHYDROUS

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A mobile colorless strongly fuming liquid. |  |
| USDOT ID Number | 1768 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 300 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Inhalation causes severe irritation of upper respiratory tract. Contact with liquid causes severe irritation of eyes and skin. Ingestion causes severe burns of mouth and stomach. |  |
| Exposure to Environment | Fumes in air. Reacts vigorously with water to form corrosive and toxic hydrofluoric acid and heat. Irritating and toxic fumes of hydrogen fluoride and phosphoric acid may be formed in fires. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – DI-N-BUTYLAMINE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A yellow-colored liquid with an amine-like odor. |  |
| USDOT ID Number | 2248 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 300 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Inhalation causes irritation of nose, throat, and lungs; coughing; nausea; headache. Ingestion causes irritation of mouth and stomach. Contact with eyes causes irritation. Contact with skin causes irritation and dermatitis. |  |
| Exposure to Environment | Flammable. Soluble in water. Produce toxic oxides of nitrogen when burned. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – GASOLINE (UNLEADED)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Clear to amber-colored volatile liquid with a petroleum-like odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewers may create fire or explosion hazards. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1203 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2mi Nighttime Isolation: 0.8mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, electric substation, Sheetz, Fast Stop Exxon, Record Delta office, Brake’s Dairy King, Dollar Tree, Breckenridge equipment lot, Pizza Hut, Lowe’s. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – LEAD DIOXIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Brown, hexagonal crystals. |  |
| USDOT ID Number | 1872 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, electric substation. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Toxic by ingestion. Inhalation of dust is toxic. Contact with substance may cause severe burns to skin and eyes. |  |
| Exposure to Environment | Fire may produce irritating, corrosive and/or toxic gases. Insoluble in water. Runoff from fire control or dilution water may cause pollution. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – LIQUID NITROGEN

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Colorless, odorless liquid. Very cold. |  |
| USDOT ID Number | 1977 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Contact of liquid with skin or eyes causes frostbite burns. Inhalation can cause asphyxiation, if atmosphere does not contain oxygen; dizziness, unconsciousness, or even death can result. Prolonged contact to equipment, etc. may cause damage. Containers may explode when burning. |  |
| Exposure to Environment | No rapid reaction with air or water. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – OXYGEN

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A colorless, odorless, tasteless gas. |  |
| USDOT ID Number | 1072 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 1,760 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, electric substation, Wal-Mart, Record Delta office, Brake’s Dairy King, Fast Stop Exxon, Sheetz, Dollar Tree, A&O railyard, Buckhannon Southern States, Burger King, Breckenridge equipment lot, Pizza Hut, Lowe’s, various residences along Cooper and Elias Streets and Leggit Road. |  |
| Susceptible Environment | Buckhannon River, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Liquid may cause frostbite of eyes and skin. Increases intensity of any fire. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Low |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – PROPANE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Colorless gas with a faint petroleum odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1075 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi | Large Spills** Initial Isolation: 2,640 ft Daytime Isolation: 1.0 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, electric substation, Wal-Mart, Record Delta office, Brake’s Dairy King, Fast Stop Exxon, Sheetz, Dollar Tree, A&O railyard, Buckhannon Southern States, Burger King, Breckenridge equipment lot, Pizza Hut, Lowe’s, Buckhannon Manor, Fred Eberle Technical Center, Cambridge Heights apartments, First Central Bank, various commercial structures on the north side of Main Street, various residences in north Buckhannon and Leggit Addition. |  |
| Susceptible Environment | Buckhannon River, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – RESIN SOLUTION

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | Liquid solution of various gums and resins in various solvents. Generally quite thick and range from clear colorless to black "tar". |  |
| USDOT ID Number | 1866 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 1.0 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, electric substation, Sheetz, Fast Stop Exxon, Record Delta office, Brake’s Dairy King, Dollar Tree, Breckenridge equipment lot, Pizza Hut, Lowe’s. |  |
| Susceptible Environment | Urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Inhalation or contact with material may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. |  |
| Exposure to Environment | Highly flammable. Vapors may form explosive mixtures with air. Some resins may have unsaturated double bonds that polymerize explosively when heated or involved in a fire. Fire may produce irritating, corrosive and/or toxic gases. Runoff from fire control may cause pollution. |  |
| Probability of Multiple Emergencies | Moderate |  |
| Unusual Conditions of Environment | None known. |  |

DETAILS – STANNIC PHOSPHIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | N/A |  |
| Quantity | N/A |  |
| Properties | A silver-white mass or lump solid. |  |
| USDOT ID Number | 1433 |  |
| Isolation Distances | Small Spills* Initial Isolation: 200 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 1.1 mi | Large Spills** Initial Isolation: 1,600 ft Daytime Isolation: 2.9 mi Nighttime Isolation: 7.0+ mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, electric substation, Wal-Mart, Record Delta office, Brake’s Dairy King, Fast Stop Exxon, Sheetz, Dollar Tree, A&O railyard, Buckhannon Southern States, Burger King, Breckenridge equipment lot, Pizza Hut, Lowe’s, various residences along Cooper and Elias Streets and Leggit Road. |  |
| Susceptible Environment | Buckhannon River, urban terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Highly toxic: contact with water produces toxic gas, may be fatal if inhaled. Inhalation or contact with vapors, substance, or decomposition products may cause severe injury or death. May produce corrosive solutions on contact with water. |  |
| Exposure to Environment | Produce flammable and toxic gases on contact with water. May ignite on contact with water or moist air. Fire will produce irritating, corrosive and/or toxic gases. Runoff from fire control may cause pollution. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | None known. |  |