LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: VIKING POOLS / CPC

| Location: | 176 Viking Drive (Lewis County Industrial Park) Jane Lew, WV 26378 |
| --- | --- |
| Owner: | Latham Plastics, Inc. 787 Watervliet Shakey Road Latham, NY 12110 |
| Emer. Contact: | Mr. Tom Marion |
| Daytime Phone: | (304) 884-6700 ext. 108 |
| 24-hr. Phone: | (304) 203-6151 |
| First Due FD: | Jane Lew Volunteer Fire Department |
| Daytime Phone: | 884-7231 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Styrene (click) |
| Lat./Long.: | 39.10269 / -80.39457 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) Aloha Analysis (click) |

DETAILS – STYRENE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Manufacturing buildings. |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | A clear, colorless to dark liquid with an aromatic odor. |  |
| USDOT ID Number | 2055 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, American Agip, PTC Alliance, Linn Operating, truck stop, other industrial park facilities. |  |
| Susceptible Environment | Low-lying areas along I-79 wherein flood waters collect during significant rain events. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents |  |
| Exposure to People/ Property | Moderate irritation of eyes and skin. High vapor concentrations may cause dizziness, drunkenness, and anesthesia. Highly flammable. |  |
| Exposure to Environment | Insoluble in water. Polymerization may take place when in fire conditions. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |