LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: WV AMERICAN WATER

| Location: | Route 2 Box 192 (in Homestead just southwest of Weston) Weston, WV 26452 |
| --- | --- |
| Owner: | WV American Water Company P. O. Box 1906 Charleston, WV 25327 |
| Emer. Contact: | Mr. Ernie Kingdon |
| Daytime Phone: | (304) 269-2006 ext. 11 |
| 24-hr. Phone: | (304) 678-7046 |
| First Due FD: | Weston Fire Department |
| Daytime Phone: | 269-2349 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Chlorine (click) |
| Lat./Long.: | 39.01512 / -80.48238 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) Aloha Analysis (click) |

DETAILS – CHLORINE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Rear left of facility (chemical storage area). |  |
| Quantity | 1,000 – 9,999 lbs |  |
| Properties | A greenish yellow gas with a pungent suffocating odor. Toxic by inhalation. Slightly soluble in water. Liquefies at -35°C and room pressure. Readily liquefied by pressure applied at room temperature. Contact with unconfined liquid can cause frostbite by evaporative cooling. Does not burn but, like oxygen, supports combustion. |  |
| USDOT ID Number | 1017 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 800 ft Daytime Isolation: 1.5 mi Nighttime Isolation: 4.6 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Highways, WV National Guard Armory, small hotel, multiple residences. |  |
| Susceptible Environment | West Fork River and public water supply. Possibly terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, sufficient safety guards in place. |  |
| Exposure to People/ Property | Nearby residents and workers can fall victim to respiratory distress or death from wind-blown vapors. When gases and liquid are given off, equipment, vehicles, etc. susceptible to corrosion may be affected. They would be repairable, however. |  |
| Exposure to Environment | Possible destruction of surrounding fauna and flora. Potential contamination of West Fork River. |  |
| Probability of Multiple Emergencies | High |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |