LEWIS UPSHUR LEPC RISK/VULNERABILITY ANALYSIS

FACILITY: WEYERHAEUSER – TRUS JOIST

| Location: | 100 TJM Drive Buckhannon, WV 26201 |
| --- | --- |
| Owner: | Weyerhaeuser Company P. O. Box 9777 Federal Way, WA 98063-9777 |
| Emer. Contact: | Mr. Walter Wagner |
| Daytime Phone: | (304) 473-5403 |
| 24-hr. Phone: | (304) 550-3064 |
| First Due FD: | Buckhannon Fire Department |
| Daytime Phone: | 472-2868 |
| 24-hr. Phone: | 911 |
| History of Events: | None to report |
| Material(s): | Clear Board Sealer Diesel Fuel (click) Emulsified Wax Ethylene Glycol (click) Propane (click) Microllam Phenolic Resin Parallam Phenolic Resin Sodium Hydroxide (click) Hydraulic Oil Thermal Oil |
| Lat./Long.: | 39.00852 / -80.20381 |
| Additional Links: | Site Map (click) Initial Isolation Distances (click) Aloha Analysis – Ethylene Glycol (click) Aloha Analysis – Propane (click) |

DETAILS – CLEAR BOARD SEALER (CBS-156-H)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Stored in microllam remanufacturing area in totes |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | White, viscous liquid with a mild, sweet aromatic odor. |  |
| USDOT ID Number | 1263 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Roadways, Weyerhaeuser facilities, residences, future industrial development areas. |  |
| Susceptible Environment | Possible terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | May irritate eyes (severely), skin, and respiratory tract. Aspiration of material into lungs may cause chemical pneumonitus and may be fatal. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – DIESEL FUEL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Log processing area; emergency fire pump house |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | A straw yellow to dark colored liquid with a petroleum-like odor. Flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Runoff into sewer may create fire or explosion hazard. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Roadways, Weyerhaeuser facilities, residences, future industrial development areas. |  |
| Susceptible Environment | Possible terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – EMULSIFIED WAX

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Parallam resin storage room (wax in mixture with resin) |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Clear liquid with slight ammonia smell. |  |
| USDOT ID Number | N/A |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.1 mi | Large Spills** Initial Isolation: 200 ft Daytime Isolation: 0.4 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Weyerhaeuser facilities and equipment. |  |
| Susceptible Environment | N/A |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Irritating to skin and eyes. Can cause respiratory irritation, dizziness, nausea, and unconsciousness. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – ETHYLENE GLYCOL

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Parallam and hydraulic rooms |  |
| Quantity | 1,000 – 9,999 lbs |  |
| Properties | Clear, colorless syrupy liquid. Primary hazard is a threat to the environment. Take immediate steps to limit spread. Can easily penetrate soil and contaminate groundwater and nearby streams. |  |
| USDOT ID Number | 1171 |  |
| Isolation Distances | Small Spills* Initial Isolation: 100 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 2,640 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Rural houses and farm land, potential future industrial operations, various Weyerhaeuser resources (equipment, vehicles). |  |
| Susceptible Environment | River and water supply, terrestrial life, possibly nearby livestock, potential for damage to nearby cemetery. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents. |  |
| Exposure to People/ Property | Irritating to skin and eyes. Can cause respiratory irritation, dizziness, nausea, and unconsciousness. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – PROPANE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Tanks located near the southwest corner of the plant site. Also tanks and cylinders on fork lifts. |  |
| Quantity | 1,000 – 9,999 lbs |  |
| Properties | Colorless gas with a faint petroleum odor. Highly flammable and easily ignited by heat, sparks, or flames. Vapor explosion hazard indoors. Vapors may travel to ignition source and flash back. Containers may explode when heated. Inhalation and contact may irritate or burn skin and eyes. Vapors may cause dizziness or suffocation. Fire may produce toxic, irritating, or corrosive gases. |  |
| USDOT ID Number | 1075 |  |
| Isolation Distances | Small Spills* Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi | Large Spills** Initial Isolation: 2,640 ft Daytime Isolation: 1.0 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Rural houses and farm land, potential future industrial operations, various Weyerhaeuser resources (equipment, vehicles). |  |
| Susceptible Environment | River and water supply, terrestrial life, possibly nearby livestock, potential for damage to nearby cemetery. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Medium (prior incident of a propane tank leak) |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness and/or suffocation from inhalation of vapors or contact. If corrosive gases are given off during fires, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – MICROLLAM PHENOLIC RESIN

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Microllam resin storage room. Phenol and formaldehyde in mixture in resins at concentrations |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Light brown to dark red, moderately viscous liquid with a phenolic odor. |  |
| USDOT ID Number | 2810 |  |
| Isolation Distances | Small Spills* Initial Isolation: 200 ft Daytime Isolation: 0.3 mi Nighttime Isolation: 1.1 mi | Large Spills** Initial Isolation: 1,100 ft Daytime Isolation: 2.1 mi Nighttime Isolation: 4.6 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Roadways, Weyerhaeuser facilities, residences, future industrial development areas. |  |
| Susceptible Environment | Possible terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents |  |
| Exposure to People/ Property | Causes burns to skin and eyes upon contact. Heated product may irritate respiratory tract. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – PARALLAM PHENOLIC RESIN

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Parallam resin room. Phenol (1%), formaldehyde (<1%), and sodium hydroxide in mixture in resins |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Light brown to dark red, moderately viscous liquid with a phenolic odor. |  |
| USDOT ID Number | 2810 |  |
| Isolation Distances | Small Spills* Initial Isolation: 200 ft Daytime Isolation: 0.3 mi Nighttime Isolation: 1.1 mi | Large Spills** Initial Isolation: 1,100 ft Daytime Isolation: 2.1 mi Nighttime Isolation: 4.6 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Roadways, Weyerhaeuser facilities, residences, future industrial development areas. |  |
| Susceptible Environment | Possible terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents |  |
| Exposure to People/ Property | Causes burns to skin and eyes upon contact. Heated product may irritate respiratory tract. |  |
| Exposure to Environment | N/A |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – SODIUM HYDROXIDE

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Log processing area near vats, parallam resin storage room (sodium hydroxide contained in resin), heat energy pump room, generator treatment chemicals |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Inhalation, ingestion, or skin contact may cause severe injury or death. Strong corrosive action on contacted tissues. |  |
| USDOT ID Number | 1824 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.1 mi Nighttime Isolation: 0.3 mi | Large Spills** Initial Isolation: 330 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Weyerhaeuser facilities and equipment. |  |
| Susceptible Environment | N/A |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents |  |
| Exposure to People/ Property | Nearby residents and workers can fall victim to respiratory distress or death from windblown dust. Severe burns to eyes and skin may result with skin contact. When gases, liquid, and dust are given off, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control may cause pollution. Dust and runoff may cause suffocation or death of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – HYDRAULIC OIL (HD 46)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Located near the southwest corner of the plant site. Various hydraulic power units are located throughout the plant (indoor and outdoor). |  |
| Quantity | 10,000 – 99,999 lbs |  |
| Properties | Pale liquid. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Roadways, Weyerhaeuser facilities, residences, future industrial development areas. |  |
| Susceptible Environment | Possible terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness, and/or suffocation from inhalation vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |

DETAILS – THERMAL OIL (R&O 46)

| GENERAL INFORMATION |  |  |
| --- | --- | --- |
| Location | Storage tank near the heat energy building, contained in the piping at heat energy, and throughout the manufacturing building. |  |
| Quantity | 100,000 – 999,999 lbs |  |
| Properties | Pale liquid. |  |
| USDOT ID Number | 1993 |  |
| Isolation Distances | Small Spills* Initial Isolation: 150 ft Daytime Isolation: 0.2 mi Nighttime Isolation: 0.8 mi | Large Spills** Initial Isolation: 1,000 ft Daytime Isolation: 0.5 mi Nighttime Isolation: 1.5 mi |
| * Small Spill – A leak from a small package or a small leak from a large package ** Large Spill – A large leak from a large package or leaks from many small packages |  |  |
| VULNERABILITY ANALYSIS |  |  |
| Susceptible Private/ Public Property | Roadways, Weyerhaeuser facilities, residences, future industrial development areas. |  |
| Susceptible Environment | Possible terrestrial life. |  |
| HAZARD ANALYSIS |  |  |
| Probability | Low, no prior incidents |  |
| Exposure to People/ Property | Persons in surrounding area may fall victim to irritated or burnt eyes and skin, dizziness, and/or suffocation from inhalation vapors or contact. If corrosive gases are given off during a fire, equipment, vehicles, etc. may be susceptible to corrosion. |  |
| Exposure to Environment | Runoff from fire control or dilution water may cause pollution. Vapors may cause suffocation of fauna. |  |
| Probability of Multiple Emergencies | Medium |  |
| Unusual Conditions of Environment | Potential flood hazard area. |  |